1. Servo Motor Control (Legs & Claw)

These pins handle the PWM signals sent to the servos to control leg and claw movements.

- GPIO 1 (D0):

  - Robot Mode: Low Left Leg / Back Left Servo (SERVO_LOW_LEFT_PIN)

  - Claw Mode: MG90S Claw Servo (SERVO_PIN)

- GPIO 2 (D1): High Right Shoulder / Front Right Servo (SERVO_HIGH_RIGHT_PIN)

- GPIO 3 (D2): High Left Shoulder / Front Left Servo (SERVO_HIGH_LEFT_PIN)

- GPIO 4 (D3): Low Right Leg / Back Right Servo (SERVO_LOW_RIGHT_PIN)

2. Ultrasonic Obstacle Sensor

These pins interface with the HC-SR04/Ultrasonic sensor to monitor safety lockouts and obstacle distances.

- GPIO 5 (D4): Trigger Pin (ULTRASONIC_TRIG_PIN)

- GPIO 6 (D5): Echo Pin (ULTRASONIC_ECHO_PIN)

3. I2S Audio Player (Speaker)

These pins handle the digital audio output (using I2S standard Philips configuration) to the external speaker/amplifier.

- GPIO 7 (D8): Word Select / Left-Right Clock (I2S_LRC_PIN)

- GPIO 8 (D9): Bit Clock (I2S_BCLK_PIN)

- GPIO 9 (D10): Data In / Audio Data (I2S_DIN_PIN)

4. Camera Controller (XIAO Sense OV2640 Board Integration)

These are internal pins routed directly through the XIAO ESP32S3 Sense board extension to interface with the camera module.

- GPIO 10: Clock (XCLK_GPIO_NUM)

- GPIO 13: Pixel Clock (PCLK_GPIO_NUM)

- GPIO 38: Vertical Sync (VSYNC_GPIO_NUM)

- GPIO 39: SCCB Clock (SIOC_GPIO_NUM)

- GPIO 40: SCCB Data (SIOD_GPIO_NUM)

- GPIO 47: Horizontal Reference (HREF_GPIO_NUM)

- Camera Data Bus Pins (Y2 to Y9):

  - GPIO 15: Y2

  - GPIO 17: Y3

  - GPIO 18: Y4

  - GPIO 16: Y5

  - GPIO 14: Y6

  - GPIO 12: Y7

  - GPIO 11: Y8

  - GPIO 48: Y9

- ---

Based on the configuration defined in your project’s source files for the ESP32-S3 Zero, here are the current pin layouts mapped by their functionality:

1. Servo Controller (Leg Joints)

These pins generate the PWM signals to drive your robot's legs:

- GPIO 9: Low Right Leg / Back Right Servo (SERVO_LOW_RIGHT_PIN)

- GPIO 10: High Right Shoulder / Front Right Servo (SERVO_HIGH_RIGHT_PIN)

- GPIO 11: High Left Shoulder / Front Left Servo (SERVO_HIGH_LEFT_PIN)

- GPIO 12: Low Left Leg / Back Left Servo (SERVO_LOW_LEFT_PIN)

2. Ultrasonic Obstacle Safety Monitor

These pins connect to your HC-SR04 ultrasonic sensor:

- GPIO 4: Trigger Pin (ULTRASONIC_TRIG_PIN)

- GPIO 5: Echo Pin (ULTRASONIC_ECHO_PIN)

3. I2S Audio Player (Speaker Output)

These pins transmit digital audio to your I2S DAC (such as the MAX98357A / HT517):

- GPIO 6: Bit Clock (I2S_BCLK_PIN)

- GPIO 7: Word Select / Left-Right Clock (I2S_LRC_PIN)

- GPIO 8: Serial Data Out (I2S_DIN_PIN)

Here is the direct migration map showing exactly how to move each wire from your ESP32-S3 Zero to the Seeed Studio XIAO ESP32S3 based on your project's codebases.

On the Seeed Studio board, the pins are typically labeled with D-labels (silkscreen) on the board itself, which map to specific raw GPIO numbers in the code.

Wire Migration Table

| Function | Robot Wire / Connection | Old Pin on ESP32-S3 Zero | New Pin on Seeed XIAO ESP32S3 |
| --- | --- | --- | --- |
| Servo Motors | Low Left Servo (Back Left) | GPIO 12 | D0 (GPIO 1) |
|  | High Left Servo (Front Left) | GPIO 11 | D2 (GPIO 3) |
|  | High Right Servo (Front Right) | GPIO 10 | D1 (GPIO 2) |
|  | Low Right Servo (Back Right) | GPIO 9 | D3 (GPIO 4) |
| Ultrasonic Sensor | Trigger (TRIG) | GPIO 4 | D4 (GPIO 5) |
|  | Echo (ECHO) | GPIO 5 | D5 (GPIO 6) |
| Audio (I2S Speaker) | Bit Clock (BCLK) | GPIO 6 | D9 (GPIO 8) |
|  | Word Select / WS (LRC) | GPIO 7 | D8 (GPIO 7) |
|  | Data In (DIN) | GPIO 8 | D10 (GPIO 9) |
| Power & Ground | VCC (5V Power) | 5V / VIN | 5V |
|  | VCC (3.3V Power) | 3V3 | 3V3 |
|  | Common Ground | GND | GND |

Step-by-Step Instructions

- Move the Servos:

  - Take the servo control wire from GPIO 12 on the S3 Zero and plug/solder it to D0 on the XIAO.

  - Move the wire from GPIO 11 on the S3 Zero to D2 on the XIAO.

  - Move the wire from GPIO 10 on the S3 Zero to D1 on the XIAO.

  - Move the wire from GPIO 9 on the S3 Zero to D3 on the XIAO.

- Move the Obstacle Sensor:

  - Move the Trigger wire from GPIO 4 on the S3 Zero to D4 on the XIAO.

  - Move the Echo wire from GPIO 5 on the S3 Zero to D5 on the XIAO.

- Move the I2S Audio Module (MAX98357A / HT517):

  - Move the Bit Clock (BCLK) wire from GPIO 6 on the S3 Zero to D9 on the XIAO.

  - Move the Word Select / Left-Right Clock (LRC / WS) wire from GPIO 7 on the S3 Zero to D8 on the XIAO.

  - Move the Data Input (DIN) wire from GPIO 8 on the S3 Zero to D10 on the XIAO.

- Connect Power and Ground:

  - Ensure the 5V, 3V3, and GND pins on your XIAO are connected to your power source/ground rails just as they were on the Zero to keep a common reference voltage.